Assistive Medication Robot Documentation

Author: Sami Berhan, Vivek Patel

Class: Introduction to Robotics, CSC 4120

Professor: Dr. Lee Yoon, Jae

- Project Overview

The Assistive Medication Robot is an end-to-end robotic system designed to assist older adults with safe and accurate medication dispensing. The system combines a mobile application, computer vision-based pill recognition, computer vision-based patient identification, and a mobile robot platform to identify a patient, store their data, and physically deliver the pill using a robotic arm. The primary goal of this project is to demonstrate the integration of robotics, embedded systems, machine learning, and software engineering into a single cohesive system suitable for real-world assistive technology applications.

- High level System Architecture

The project is divided into four major subsystems:

- Mobile Application

- Backend/Logic Layer

- Computer Vision & Machine Learning

- Hardware & Control

Each subsystem is modular allowing independent testing and future expansion.

- Mobile Application

3.1 Purpose

The mobile app serves as the primary user interface for caregivers and patients. It allows users to:

- View medication schedules

- Identify which pill should be administered

- Sends commands to the robot

- Receive alerts and confirmations from the robot

3.2 Technology Stack

- Framework: React

- Language: JavaScript

- Platform: web based and mobile friendly UI

- Database: MongoDB

- FastAPi for communication between the robot and the app

3.3 Core App Features

- User Authentication

- Medication Schedule Management

I.  Pill Name

II. Dosage

III. Time of administration

- Robot Control Interface

I. Send dispense command

II. Monitor robot status

- Notification

3.4 App robot communication

The app communicates with the robot through a backend service layer sending high level commands such as:

- START_DISPENSE

- VERIFY_PILL

- DELIVERY_COMPLETE

- Backend and logic layer

4.1 Purpose

The backend serves as the central coordination layer between the React application, the computer vision pipeline, and the robotic hardware. It is responsible for handling API requests, managing data persistence, and orchestrating system wide logic.

4.2  Technology Stack

- Framework: FastAPI

- API Style: RESTful API

- Database: MongoDB

- ODM: PyMongo

- Authentication: Token-based

4.3 Core Responsibilities

- Handle requests from the React application

- Validate medication schedules and permissions

- Store and retrieve patient, medication, and event data

- Trigger computer vision

- Send commands to robotic control scripts

- Log dispensing events for auditing and safety

4.4 Data Flow

1. React app sends a dispense request

2. Fast API validates request and schedule

3. MongoDB queried for patient and medication data

4. Vision pipeline verifies the pill

5. Robot executes dispensing sequence

6. Result stored in MongoDB and returned to app

- Computer Vision & Machine Learning

5.1 Objective

The computer vision system is responsible for pill recoginition and verification before dispensing.

5.2 Dataset

- Dataset used: C3PI and Kaggle pill recognition

- Multiple pill shapes, colors, and markings

- Images caputered ucer varying lighting conditions

5.3 Model Pipeline

- Image preprocessing(resize, normalization)

- CNN based pill classifier

- Inference run locally on the Raspberry Pi

5.4 Output

The model outputs:

- Pill class ID

- Confidence score

Dispensing proceeds only if the detected pill matches the scheduled medication with sufficient confidence.

- Robotics Hardware

6.1 Core Components

- Robot Base: GoPiGo3

- Compute: Raspberry Pi5

- Camera: Raspberry Pi Camera Module 3

- Robotic Arm: 4 DOF arm

- Servos: MG996R

- Servo Driver: PCA9685(I2C)

- Sensors: Ultrasonic distance sensor

6.2 Hardware Responsibilities

- Navigate to dispensing position

- Position robotic arm accurately

- Pick and place pill

- Avoid collision

6.3 Servo Control

- Controlled via PCA96585 over I2C

- Each join mapped to a spefic channel

- Smooth motion profiles to prevent pill damage

- Software on Raspberry pi 5

7.1 Environmental

- Operating system: Pi OS Bookworm

- Language: Python 3

- Libraries: Open cv, Numpy, GoPiGo3 drives

7.2 Responsibilities

- Camera access and live feed

- Running ML inference

- Motor and servo control

- Local testing and debugging

7.3 Testing Modes

- Laptop Mode: vision model tested using webcam

- Pi Mode: Full system integration

- Safety and Reliability Considerations

- Pill verification before dispensing

- Confidence threshold for ML predictions

- Manual override via app

- Logging of all dispensing events

- Challenges Encountered

- Raspberry Pi5 and GoPiGo3 compatibility

- Camera driver issues on Bookworm OS

- Limited ML experience

- Limited Hardware Knowledge

- Real time coordination between vision and hardware

Each challenge was addressed through modular testing

- Future Improvements

- ROS2 integration for advanced autonomy

- Multi pill sorting

- Cloud based model updates

- Conclusion

The project successfully demonstrates a full stack assistive robotics system integrating:

- Mobile application development

- Embedded systems

- Computer vision and machine learning

- Robotic hardware control

The Assistive Medication Robot highlights how interdisciplinary engineering can be applied to solve real world healthcare challenges and serves as a strong foundation for future academic research and professional development.